A Dictionary of Measures, Units and Conversions

A Dictionary of Units

--------------------------------------------------------------------------------

The Systeme International [S I]

Le Systeme international d'Unites officially came into being in October 1960

and has been adopted by nearly all countries, though the amount of actual usage

varies considerably.

It is based upon 7 principal units, 1 in each of 7 different categories -

Category             Name            Abbreviation

Length                  metre                   m

Mass                    kilogram                kg

Time                    second                  s

Electric current        ampere                  A

Temperature             kelvin                  K

Amount of substance     mole                    mol

Luminous intensity      candela                 cd

Definitions of these basic units are given.

Each of these units may take a prefix.

From these basic units many other units are derived and named.

--------------------------------------------------------------------------------

Definitions of the Seven Basic S I Units

metre [m]

The metre is the basic unit of length. It is the distance light

travels, in a vacuum, in 1/299792458th of a second.

kilogram [kg]

The kilogram is the basic unit of mass. It is the mass of an

international prototype in the form of a platinum-iridium cylinder kept

at Sevres in France. It is now the only basic unit still defined in

terms of a material object, and also the only one with a prefix[kilo]

already in place.

second [s]

The second is the basic unit of time. It is the length of time taken

for 9192631770 periods of vibration of the caesium-133 atom to occur.

ampere [A]

The ampere is the basic unit of electric current. It is that current

which produces a specified force between two parallel wires which are 1

metre apart in a vacuum.It is named after the French physicist Andre

Ampere (1775-1836).

kelvin [K]

The kelvin is the basic unit of temperature. It is 1/273.16th of the

thermodynamic temperature of the triple point of water. It is named

after the Scottish mathematician and physicist William Thomson 1st Lord

Kelvin (1824-1907).

mole [mol]

The mole is the basic unit of substance. It is the amount of substance

that contains as many elementary units as there are atoms in 0.012 kg

of carbon-12.

candela [cd]

The candela is the basic unit of luminous intensity. It is the

intensity of a source of light of a specified frequency, which gives a

specified amount of power in a given direction.

--------------------------------------------------------------------------------

Derived Units of the S I

From the 7 basic units of the SI many other units are derived for a variety of

purposes. Only some of them are explained here. The units printed in bold are

either basic units or else, in some cases, are themselves derived.

farad [F]

The farad is the SI unit of the capacitance of an electrical system,

that is, its capacity to store electricity. It is a rather large unit

as defined and is more often used as a microfarad. It is named after

the English chemist and physicist Michael Faraday (1791-1867).

hertz [Hz]

The hertz is the SI unit of the frequency of a periodic phenomenon. One

hertz indicates that 1 cycle of the phenomenon occurs every second. For

most work much higher frequencies are needed such as the kiloherz [kHz]

and megaherz [MHz]. It is named after the German physicist Heinrich

Rudolph Herz (1857-94).

joule [J]

The joule is the SI unit of work or energy. One joule is the amount of

work done when an applied force of 1 newton moves through a distance of

1 metre in the direction of the force.It is named after the English

physicist James Prescott Joule (1818-89).

newton [N]

The newton is the SI unit of force. One newton is the force required to

give a mass of 1 kilogram an acceleration of 1 metre per second per

second. It is named after the English mathematician and physicist Sir

Isaac Newton (1642-1727).

ohm [*]

The ohm is the SI unit of resistance of an electrical conductor. Its

symbol, shown here as [*] is the Greek letter known as 'omega'. It is

named after the German physicist Georg Simon Ohm (1789-1854).

pascal [Pa]

The pascal is the SI unit of pressure. One pascal is the pressure

generated by a force of 1 newton acting on an area of 1 square metre.

It is a rather small unit as defined and is more often used as a

kilopascal [kPa]. It is named after the French mathematician, physicist

and philosopher Blaise Pascal (1623-62).

volt [V]

The volt is the SI unit of electric potential. One volt is the

difference of potential between two points of an electical conductor

when a current of 1 ampere flowing between those points dissipates a

power of 1 watt. It is named after the Italian physicist Count

Alessandro Giuseppe Anastasio Volta (1745-1827).

watt [W]

The watt is used to measure power or the rate of doing work. One watt

is a power of 1 joule per second. It is named after the Scottish

engineer James Watt (1736-1819).

Note that prefixes may be used in conjunction with any of the above units.

--------------------------------------------------------------------------------

The Prefixes of the S I

The S I allows the sizes of units to be made bigger or smaller by the use of

appropriate prefixes. For example, the electrical unit of a watt is not a big

unit even in terms of ordinary household use, so it is generally used in terms

of 1000 watts at a time. The prefix for 1000 is kilo so we use kilowatts[kW] as

our unit of measurement. For makers of electricity, or bigger users such as

industry, it is common to use megawatts[MW] or even gigawatts[GW]. The full

range of prefixes with their [symbols or abbreviations] and their multiplying

factors which are also given in other forms is

yotta [Y] 1 000 000 000 000 000 000 000 000     = 10^24

zetta [Z] 1 000 000 000 000 000 000 000         = 10^21

exa   [E] 1 000 000 000 000 000 000             = 10^18

peta  [P] 1 000 000 000 000 000                 = 10^15

tera  [T] 1 000 000 000 000                     = 10^12

giga  [G] 1 000 000 000                    (a thousand millions = a billion)

mega  [M] 1 000 000                        (a million)

kilo  [k] 1 000                            (a thousand)

hecto [h] 100

deca  [da]10

1

deci  [d] 0.1

centi [c] 0.01

milli [m] 0.001                            (a thousandth)

micro [u] 0.000 001                          (a millionth)

nano  [n] 0.000 000 001                    (a thousand millionth)

pico  [p] 0.000 000 000 001                     = 10^-12

femto [f] 0.000 000 000 000 001                 = 10^-15

atto  [a] 0.000 000 000 000 000 001             = 10^-18

zepto [z] 0.000 000 000 000 000 000 001         = 10^-21

yocto [y] 0.000 000 000 000 000 000 000 001     = 10^-24

[u] the symbol used for micro is the Greek letter known as 'mu'

Nearly all of the S I prefixes are multiples or sub-multiples of 1000. However,

these are inconvenient for many purposes and so hecto, deca, deci, and centi

are also used.

deca also appears as deka [da] or [dk] in the USA and Contintental Europe. So

much for standards!

--------------------------------------------------------------------------------

Conventions of Usage in the S I

There are various rules laid down for the use of the SI and its units as well

as some observations to be made that will help in its correct use.

Any unit may take only ONE prefix. For example 'millimillimetre' is

incorrect and should be written as 'micrometre'.

Most prefixes which make a unit bigger are written in capital letters

(M G T etc.), but when they make a unit smaller then lower case (m n p

etc.) is used. Exceptions to this are the kilo [k] to avoid any

possible confusion with kelvin [K]; hecto [h]; and deca [da] or [dk]

A unit which is named after a person is written all in lower case

(newton, volt, pascal etc.) when named in full, but starting with a

capital letter (N V Pa etc.) when abbreviated. An exception to this

rule is the litre which, if written as a lower case 'l' could be

mistaken for a '1' (one) and so a capital 'L' is allowed as an

alternative. It is intended that a single letter will be decided upon

some time in the future when it becomes clear which letter is being

favoured most in use.

Units written in abbreviated form are NEVER pluralised. So 'm' could

always be either 'metre' or 'metres'. 'ms' could represent 'metre

second' (whatever that is) or, more correctly, 'millisecond'.

An abbreviation (such as J N g Pa etc.) is NEVER followed by a

full-stop unless it is the end of a sentence.

To make numbers easier to read they may be divided into groups of 3

separated by spaces (or half-spaces) but NOT commas.

The SI preferred way of showing a decimal fraction is to use a comma

(123,456) to separate the whole number from its fractional part. The

practice of using a point, as is common in English-speaking countries,

is acceptable providing only that the point is placed ON the line of

the bottom edge of the numbers (123.456).

It will be noted that many units are eponymous, that is they are named

after persons. This is always someone who was prominent in the early

work done within the field in which the unit is used.

--------------------------------------------------------------------------------

A Brief History of Measurement

One of the earliest types of measurement concerned that of length. These

measurements were usually based on parts of the body. A well documented example

(the first) is the Egyptian cubit which was derived from the length of the arm

from the elbow to the outstretched finger tips. By 2500 BC this had been

standardised in a royal master cubit made of black marble (about 52 cm). This

cubit was divided into 28 digits (roughly a finger width) which could be

further divided into fractional parts, the smallest of these being only just

over a millimetre.

In England units of measurement were not properly standardised until the 13th

century, though variations (and abuses) continued until long after that. For

example, there were three different gallons (ale, wine and corn) up until 1824

when the gallon was standardised.

In the U S A the system of weights and measured first adopted was that of the

English, though a few differences came in when decisions were made at the time

of standardisation in 1836. For instance, the wine-gallon of 231 cubic inches

was used instead of the English one (as defined in 1824) of about 277 cubic

inches. The U S A also took as their standard of dry measure the old Winchester

bushel of 2150.42 cubic inches, which gave a dry gallon of nearly 269 cubic

inches.

Even as late as the middle of the 20th century there were some differences in

UK and US measures which were nominally the same. The UK inch measured 2.53998

cm while the US inch was 2.540005 cm. Both were standardised at 2.54 cm in July

1959, though the U S continued to use 'their' value for several years in land

surveying work - this too is slowly being metricated.

In France the metric system officially started in June 1799 with the declared

intent of being 'For all people, for all time'. The unit of length was the

metre which was defined as being one ten-millionth part of a quarter of the

earth's circumference. The production of this standard required a very careful

survey to be done which took several years. However, as more accurate

instruments became available so the 'exactness' of the standard was called into

question. Later efforts were directed at finding some absolute standard based

on an observable physical phenomenon. Over two centuries this developed into

the S I. So maybe their original slogan was more correct than anyone could have

foreseen then.

--------------------------------------------------------------------------------

Metric System of Measurements

Length                                         Area

10 millimetres = 1 centimetre                 100 sq. mm     = 1 sq. cm

10 centimetres = 1 decimeter               10 000 sq. cm     = 1 sq. metre

10 decimetres  = 1 metre                      100 sq. metres = 1 are

10 metres      = 1 decametre                  100 ares       = 1 hectare

10 decametres  = 1 hectometre              10 000 sq. metres = 1 hectare

10 hectometres = 1 kilometre                  100 hectares   = 1 sq. kilometre

1000 metres      = 1 kilometre            1 000 000 sq. metres = 1 sq. kilometre

Volume                                           Capacity

1000 cu. mm = 1 cu. cm                           10 millilitres = 1 centilitre

1000 cu. cm = 1 cu. decimetre                    10 centilitree = 1 decilitre

1000 cu. dm = 1 cu. metre                        10 decilitres  = 1 litre

1 million cu. cm = 1 cu. metre                      1000 litres      = 1 cu. metre

Mass

1000 grams     = 1 kilogram

1000 kilograms = 1 tonne

The distinction between 'Volume' and 'Capacity' is artificial and kept here

only for historic reasons.

A millitre is a cubic centimetre and a cubic decimetre is a litre. But see

under 'Volume' for problems with the litre.

--------------------------------------------------------------------------------

The U K (Imperial) System of Measurements

Length                                      Area

12 inches   = 1 foot                   144 sq. inches = 1 square foot

3 feet     = 1 yard                     9 sq. feet   = 1 square yard

22 yards    = 1 chain                 4840 sq. yards  = 1 acre

10 chains   = 1 furlong                640 acres      = 1 square mile

8 furlongs = 1 mile

5280 feet     = 1 mile

1760 yards    = 1 mile                          Capacity

20 fluid ounces = 1 pint

Volume                            4 gills        = 1 pint

1728 cu. inches = 1 cubic foot           2 pints        = 1 quart

27 cu. feet   = 1 cubic yard           4 quarts       = 1 gallon (8 pints)

Mass (Avoirdupois)

437.5 grains = 1 ounce                          Troy Weights

16 ounces   = 1 pound (7000 grains)    24 grains        = 1 pennyweight

14 pounds   = 1 stone                  20 pennyweights  = 1 ounce (480 grains)

8 stones   = 1 hundredweight [cwt]    12 ounces        = 1 pound (5760 grains)

20 cwt      = 1 ton (2240 pounds)

Apothecaries' Measures                  Apothecaries' Weights

20 minims      = 1 fl.scruple          20 grains   = 1 scruple

3 fl.scruples = 1 fl.drachm            3 scruples = 1 drachm

8 fl.drachms  = 1 fl.ounce             8 drachms  = 1 ounce (480 grains)

20 fl.ounces   = 1 pint                12 ounces   = 1 pound (5760 grains)

The old Imperial (now UK) system was originally defined by three standard

measures - the yard, the pound and the gallon which were held in London. They

are now defined by reference to the S I measures of the metre, the kilogram and

the litre. These equivalent measures are exact.

1 yard = 0.9144 metres - same as US

1 pound = 0.453 592 37 kilograms - same as US

1 gallon = 4.546 09 litres

Note particularly that the UK gallon is a different size to the US gallon so

that NO liquid measures of the same name are the same size in the UK and US

systems.

Also that the ton(UK) is 2240 pounds while a ton(US) is 2000 pounds. These are

also referred to as a long ton and short ton respectively.

--------------------------------------------------------------------------------

Metrication in the U K

There have been three major Weights and Measures Acts in recent times (1963,

1976 and 1985) all gradually abolishing various units, as well re-defining the

standards. All the Apothecaries' measures are gone, and of the Troy measures,

only the ounce remains. Currently legislation has decreed that -

From the 1st October 1995, for economic, public health, public safety and

administrative purposes, only metric units are allowed EXCEPT that -

* pounds and ounces for weighing of goods sold from bulk

* pints and fluid ounces for beer, cider, waters, lemonades and fruit juices

in RETURNABLE containers

* therms for gas supply

* fathoms for marine navigation

may be used until 31st December 1999.

The following may continue to be used WITHOUT time limit -

* miles, yards, feet and inches for road traffic signs and related

measurements of speed and distance

* pints for dispensing draught beer and cider, and for milk in RETURNABLE

containers

* acres for land registration purposes

* troy ounces for transactions in precious metals.

Sports are exempt from all of this, but most of them have (voluntarily) changed

their relevant regulations into statements of equivalent metric measures.

That is how the legislation is framed. In common usage the 'old' units are

still very apparent.

--------------------------------------------------------------------------------

The U S System of Measurements

Most of the US system of measurements is the same as that for the UK. The

biggest differences to be noted are in Capacity which has both liquid and dry

measures as well as being based on a different standard - the US liquid gallon

is smaller than the UK gallon. There is also a measurement known at the US

survey foot. It is gradually being phased out as the maps and land plans are

re-drawn under metrication. (The changeover is being made by putting 39.37 US

survey feet = 12 metres)

Length                                   Area

12 inches   = 1 foot                   144 sq. inches = 1 square foot

3 feet     = 1 yard                     9 sq. feet   = 1 square yard

220 yards    = 1 furlong               4840 sq. yards  = 1 acre

8 furlongs = 1 mile                   640 acres      = 1 square mile

5280 feet     = 1 mile                     1 sq.mile    = 1 section

1760 yards    = 1 mile                    36 sections   = 1 township

Volume

1728 cu. inches = 1 cubic foot

27 cu. feet   = 1 cubic yard

Capacity (Dry)                           Capacity (Liquid)

16 fluid ounces = 1 pint

2 pints   = 1 quart                     4 gills        = 1 pint

8 quarts  = 1 peck                      2 pints        = 1 quart

4 pecks   = 1 bushel                    4 quarts       = 1 gallon (8 pints)

Mass

437.5 grains = 1 ounce                          Troy Weights

16 ounces   = 1 pound (7000 grains)    24 grains        = 1 pennyweight

14 pounds   = 1 stone                  20 pennyweights  = 1 ounce (480 grains)

100 pounds   = 1 hundredweight [cwt]    12 ounces        = 1 pound (5760 grains)

20 cwt      = 1 ton (2000 pounds)

Apothecaries' Measures         Apothecaries' Weights

60 minims    = 1 fl.dram               20 grains   = 1 scruple

8 fl.drams  = 1 fl.ounce               3 scruples = 1 dram

16 fl.ounces = 1 pint                   8 drams    = 1 ounce (480 grains)

12 ounces   = 1 pound (5760 grains)

As with the UK system these measures were originally defined by physical

standard measures - the yard, the pound, the gallon and the bushel.They are now

all defined by reference to the S I measures of the metre, the kilogram and the

litre. These equivalent measures are exact.

1 yard = 0.9144 metres - same as UK

1 pound = 0.453 592 37 kilograms - same as UK

1 gallon (liquid) = 3.785 411 784 litres

1 bushel = 35.239 070 166 88 litres

Note particularly that the US gallon is a different size to the UK gallon so

that NO liquid measures of the same name are the same size in the US and UK

systems.

Also that the ton(US) is 2000 pounds while a ton(UK) is 2240 pounds. These are

also referred to as a short ton and long ton respectively.

Note than in matters concerned with land measurements, for the most accurate

work, it is necessary to establish whether the US survey measures are being

used or not.

--------------------------------------------------------------------------------

A

acres angstroms ares astronomical units atmospheres

B

barleycorns barrels (oil) bars British thermal units Btu/hour etc. bushels

C

calories calories per hour etc. carats, metric centigrade heat units

centilitres centimetres centimetres of mercury or water centimetres per minute

etc. chains (surveyors') circular inches cubic (+ any units) cubic measures per

area cubits

D

decilitres denier drex dynes

E

ells (UK) ems (pica) ergs (energy) ergs (torque)

F

fathoms feet feet of water feet per hour etc. fluid ounces foot pounds-force

foot pounds-force per minute etc. foot poundals furlongs

G

gallons gallons per area gigajoules gigawatts grains grains per gallon grams

gram-force centimetres grams per area grams per cm grams per (any volume)

H

hands hectares hides horsepower horsepower hours hundredweights

IJ

inches inches of mercury or water inches of rain (by mass) inches of rain (by

volume) inches per minute etc. joules joules per hour etc.

K

kilocalories kilocalories per hour etc. kilograms-force kilogram-force metres

(energy) kilogram-force metres (torque) kilogram-force metres per hour etc.

kilogram-force per area kilograms kilograms per area kilograms per metre

kilograms per volume kilojoules kilojoules per hour etc. kilometres kilometres

per hour etc. kilometres per litre kilonewton per square metre kilonewtons

kilopascals kilowatts kilowatt hours kips (weight) kips (force) kips per square

inch knots

L

leagues light years links (surveyors') litres

litres per area

M

Mach number megajoules meganewtons meganewtons per square metre megawatts metres

metres of water metres per second etc. microns (=micrometres) miles miles per

gallon miles per hour etc.

millibars milligrams per cm milligrams per (any volume) millilitres millimetres

of mercury or water millimetres of rain (by mass) millimetres of rain (by

volume)

N

newton metres (energy) newton metres (torque) newtons (per area) newtons

(force) newtons (weight)

O

ounces

ounces per inch ounces per area ounces per volume

PQ

parsecs pascals perch (=rods or poles) picas pints points (printers') poundals

poundals per square foot pounds pounds per area pounds per foot pounds per

volume pounds-force pound-force inches pounds-force per area quarts

R

roods

S

slugs (or g-pounds) stones square (+ any units) squares (of timber) sthenes

T

tex therms tonnes ton-force metres tonnes-force tonnes-force per area tonnes

per hectare tonnes per km tonnes per volume ton-force feet tons tons-force

tons-force per area tons per acre tons per mile tons per volume townships troy

ounce

UVW

watt second watt hours watts

XYZ

yards yards per hour etc.

--------------------------------------------------------------------------------

Length

The S I unit of length is the metre. To change any of these other units of

length into their equivalent values in metres use the operation and conversion

factor given. Those marked with # are exact. Other values are given to an

appropriate degree of accuracy. Where some uncertainty is indicated it means

that a good idea of the size of the unit can be given but that a better value

would depend upon knowing the period and/or culture in which the unit was being

used.

angstroms divide by 10 000 000 000 #

astronomical units      x 149 598 550 000

barleycorns             x 0.008 467

centimetres             x 0.01 #

chains (surveyors')     x 20.1168 #

cubits                  x (0.45 to 0.5)

ells (UK)               x 0.875 (but many variations)

ems (pica)              x 0.004 233 3

fathoms                 x 1.8288 #

feet (UK and US)        x 0.3048 #

feet (US survey)        x 0.304 800 609 6

furlongs                x 201.168 #

hands                   x 0.106 #

inches                  x 0.0254 #

kilometres              x 1000 #

leagues                 x (4000 to 5000)

light years             x 9 460 500 000 000 000

links (surveyors')      x 0.201 168 #

metres [m]              1

microns (=micrometres)  x 0.000 001 #

miles (UK and US)       x 1609.344 #

miles (nautical)        x 1852 #

parsecs                 x 30 856 770 000 000 000

perch (=rods or poles)  x 5.0292 #

picas (computer)        x 0.004 233 333

picas (printers')       x 0.004 217 518

points (computer)       x 0.000 352 777 8

points (printers')      x 0.000 351 459 8

yards                   x 0.9144 #

Note than in matters concerned with land measurements, for the most accurate

work, it is necessary to establish whether the US survey measures are being

used or not.

--------------------------------------------------------------------------------

Area

The S I unit of area is the square metre. To change any of these other units of

area into their equivalent values in square metres use the operation and

conversion factor given. Those marked with # are exact. Other values are given

to an appropriate degree of accuracy. Where some uncertainty is indicated it

means that a good idea of the size of the unit can be given but that a better

value would depend upon knowing the period and/or culture in which the unit was

being used.

acres x 4046.856 422 4 #

ares                    x 100 #

circular inches         x 0.000 506 707 479

hectares                x 10 000 #

hides                   x 485 000 (with wide variations)

roods                   x 1011.714 105 6 #

square centimetres      x 0.000 1 #

square feet (UK and US) x 0.092 903 04 #

square feet (US survey) x 0.092 903 411 613

square inches           x 0.000 645 16 #

square kilometres       x 1 000 000 #

square metres           1

square miles            x 2 589 988.110 336 #

square millimetres      x 0.000 001 #

squares (of timber)     x 9.290 304 #

square rods (or poles)  x 25.292 852 64 #

square yards            x 0.836 127 36 #

townships               x 93 239 571.972

Note than in matters concerned with land measurements, for the most accurate

work, it is necessary to establish whether the US survey measures are being

used or not.

--------------------------------------------------------------------------------

Volume or Capacity

The S I unit of volume is the cubic metre. However, this seems to be much less

used than the litre (1000 litres = 1 cubic metre).To change any of these other

units of volume into their equivalent values in litres use the operation and

conversion factor given. Those marked with # are exact. Other values are given

to an appropriate degree of accuracy.

The litre. There can be some ambiguity about the size of the litre. In 1901 it

was defined by reference to a kilogram of pure water under certain particular

conditions. (This was similar to the way the old UK gallon was set.) In 1964 it

was re-defined as a common usage term for a cubic decimetre. They differ very

slightly and for really accurate work, to avoid any possible confusion, it is

recommended that the litre is not used . It is used here as being a cubic

decimetre.

barrels (oil) x 158.987 294 928 #

bushels (UK)            x 36.368 72 #

bushels (US)            x 35.239 070 166 88 #

centilitres             x 0.01 #

cubic centimetres       x 0.001 #

cubic decimetres        1

cubic decametres        x 1 000 000 #

cubic feet              x 28.316 846 592 #

cubic inches            x 0.016 387 064 #

cubic metres            x 1000 #

cubic millimetres       x 0.000 001 #

cubic yards             x 764.554 857 984 #

decilitres              x 0.1 #

fluid ounces (UK)       x 0.028 413 062 5 #

fluid ounces (US)       x 0.029 573 534 562 5 #

gallons (UK)            x 4.546 09 #

gallons, dry (US)       x 4.404 883 770 86 #

gallons, liquid (US)    x 3.785 411 784 #

litres [l or L]         1

litres (1901 - 1964)    x 1.000 028

millitres               x 0.001 #

pints (UK)              x 0.568 261 25 #

pints, dry (US)         x 0.550 610 471 357 5 #

pints, liquid (US)      x 0.473 176 473 #

quarts (UK)             x 1.136 522 5 #

quarts, dry (US)        x 1.101 220 942 715 #

quarts, liquid (US)     x 0.946 352 946 #

--------------------------------------------------------------------------------

Mass (or Weight)

The S I unit of mass is the kilogram. To change any of these other units of

mass into their equivalent values in kilograms use the operation and conversion

factor given. Those marked with # are exact. Other values are given to an

appropriate degree of accuracy.

carats, metric x 0.000 2 #

grains                  x 0.000 064 798 91 #

grams                   x 0.001 #

hundredweights, long    x 50.802 345 44 #

hundredweights, short   x 45.359 237 #

kilograms [kg]          1

ounces, avoirdupois     x 0.028 349 523 125 #

ounces, troy            x 0.031 103 476 8 #

pounds                  x 0.453 592 37 #

slugs (or g-pounds)     x 14.593 903

stones                  x 6.350 293 18 #

tons (UK or long)       x 1016.046 908 8 #

tons (US or short)      x 907.184 74 #

tonnes                  x 1000 #

--------------------------------------------------------------------------------

Line density

Line density is a measure of mass per unit length. The S I compatible unit of

line density is kilograms/metre. A major use of line density is in the textile

industry to indicate the coarseness of a yarn or fibre. For that purpose the SI

unit is rather large so the preferred unit there is the tex. (1 tex = 1

gram/kilometre) To change any of these other units of line density into their

equivalent values in kilograms/metre use the operation and conversion factor

given. Those marked with # are exact. Other values are given to an appropriate

degree of accuracy.

denier divide by 9 000 000 #

drex                    divide by 10 000 000 #

grams/centimetre        divide by 10 #

grams/kilometre (tex)   divide by  1 000 000 #

grams/metre             divide by 1000 #

grams/millimetre        1

kilograms/kilometre     divide by 1000 #

kilograms/metre      1

milligrams/centimetre   divide by 10 000 #

milligrams/millimetre   divide by   1000 #

ounces/inch             x 1.116 125

ounces/foot             x 0.093 01

pounds/inch             x 17.858

pounds/foot             x 1.488 164

pounds/yard             x 0.496 055

pounds/mile             x 0.000 281 849

tex                     divide by 1 000 000 #

tons(UK)/mile           x 0.631 342

tons(US)/mile           x 0.563 698

tonnes/kilometre        1

--------------------------------------------------------------------------------

Density

Density is the shortened term generally used in place of the more accurate

description volumetric density.It is a measure of mass per unit volume. The S I

compatible unit of density is kilograms/cubic metre. However, this a rather

large unit for most purposes (iron is over 7000, wood is about 600 and even

cork is over 200). A much more useful size of unit is kilograms/litre (for

which the previous values then become 7, 0.6 and 0.2 respectively). This unit

also has the great advantage of being numerically unchanged for grams/cubic

centimetre and tonnes/cubic metre (or megagrams/cubic metre). To change any of

these other units of density into their equivalent values in kilograms/litre

use the operation and conversion factor given. Those marked with # are exact.

Other values are given to an appropriate degree of accuracy.

grains/gallon(UK) divide by 70 156

grains/gallon(US)       divide by  58 418

grams/cubic centimetre  1

grams/litre             divide by 1000 #

grams/millilitre        1

kilograms/cubic metre   divide by 1000 #

megagrams/cubic metre   1

milligrams/millitre     divide by 1000 #

milligrams/litre        divide by 1 000 000 #

kilograms/litre      1

ounces/cubic inch       x 1.729 994 044

ounces/gallon(UK)       x 0.006 236 023

ounces/gallon(US)       x 0.007 489 152

pounds/cubic inch       x 27.679 904

pounds/cubic foot       x 0.016 018 463

pounds/gallon(UK)       x 0.099 776 373

pounds/gallon(US)       x 0.119 826 427

tonnes/cubic metre      1

tons(UK)/cubic yard     x 1.328 939 184

tons(US)/cubic yard     x 1.186 552 843

--------------------------------------------------------------------------------

Energy or work

There is a lot of room for confusion in some of the units used here. The

calorie can take 5 different values and, while these do not vary by very much,

for accurate work it is necessary to specify which calorie is being used.

The 5 calories are known as the International Table calorie - cal(IT); the

thermochemical calorie - cal(th); the mean calorie - cal(mean); the 15 degree C

calorie - cal(15C); and the 20 degree C calorie - cal(20C).

As a further complication, in working with food and expressing nutritional

values, the unit of a Calorie (capital C) is often used to represent 1000

calories, and again it is necessary to specify which calorie is being used for

that.

The British thermal unit (Btu) can also take different values and they are

named in a similar way to the calorie, that is Btu (IT), (th), etc. Also note

that the therm is 100 000 Btu so its exact size depends on which Btu is being

used.

The S I unit of energy or work is the joule. To change any of these other units

of energy or work into their equivalent values in joules use the operation and

conversion factor given. Those marked with # are exact. Other values are given

to an appropriate degree of accuracy.

British thermal units(IT)x 1055.056

Btu (th)    x 1054.350

Btu (mean)  x 1055.87

calories   - cal (IT)    x 4.1868 #

- cal (th)    x 4.184 #

- cal (mean)  x 4.190 02

- cal (15C)   x 4.185 80

- cal (20C)   x 4.181 90

Calorie (food)           x 4186 (approx.)

centigrade heat units    x 1900.4

ergs                     divide by 10 000 000 #

foot pounds-force        x 1.355 817

foot poundals            x 0.042 140

gigajoules [GJ]          x 1000 000 000 #

horsepower hours         x 2 684 520 (approx.)

joules [J]               1

kilocalories (IT)        x 4186.8 #

kilocalories (th)        x 4184 #

kilogram-force metres    x 9.806 65 #

kilojoules [kJ]          x 1000 #

kilowatt hours [kWh]     x 3 600 000 #

megajoules [MJ]          x 1 000 000 #

newton metres [Nm]       x 1 #

therms                   x 105 500 000 (approx.)

watt seconds [Ws]        1

watt hours [Wh]          x 3600 #

--------------------------------------------------------------------------------

Force

The S I unit of force is the newton. To change any of these other units of

force into their equivalent values in newtons use the operation and conversion

factor given. Those marked with # are exact. Other values are given to an

appropriate degree of accuracy.

dynes divide by 100 000 #

kilograms force         x 9.806 65 #

kilonewtons [kN]        x 1000 #

kips                    x 4448.222

meganewtons [MN]        x 1 000 000 #

newtons [N]          1

pounds force            x 4.448 222

poundals                x 0.138 255

sthenes (=kN)           x 1000

tonnes force            x 9806.65 #

tons(UK) force          x 9964.016

tons(US) force          x 8896.443

--------------------------------------------------------------------------------

Fuel Consumption

Fuel consumption of any means of transport (car, aeroplane, ship etc.) that

uses fuel is a measure giving the relationship between the distance travelled

for an amount of fuel used. The most common example is the car where it is

usually expressed (in English-speaking countries) in miles per gallon.

It could also be expressed in gallons per mile. However, for a car the latter

method gives a rather small figure: 35 miles per gallon is about 0.0286 gallons

per mile. In that case it would be better to give a figure for 100 miles, so it

would be 2.86 gallons per 100 miles. That is the metric way of expressing fuel

consumption - as litres per 100 kilometres.

From regular enquiries it appears that in real life people are using all sorts

of ways of expressing their fuel consumption, so this section (unlike all the

others) tries to cover as many ways as possible. All the values are given to an

accuracy of 4 significant figures.

To change                       into

miles per gallon (UK)           miles per gallon (US) multiply by 0.833

miles per gallon (UK)           miles per litre multiply by 0.22

miles per litre                 miles per gallon (UK) multiply by 4.456

miles per gallon (UK)           kilometres per litre multiply by 0.354

miles per gallon (US)           miles per gallon (UK) multiply by 1.2

miles per gallon (US)           miles per litre multiply by 0.2642

miles per litre                 miles per gallon (US) multiply by 3.785

miles per gallon (US)           kilometres per litre multiply by 0.4251

X miles per gallon               gallons per 100 miles: divide 100 by X

(both gallons must of the same type)

X miles per gallon (UK)          litres per 100 km: divide 282.5 by X

X miles per gallon (US)          litres per 100 km: divide 235.2 by X

X km per litre                   litres per 100 km: divide 100  by X

X miles per litre                litres per 100 km: divide 62.14  by X

--------------------------------------------------------------------------------

Power

Since power is a measure of the rate at which work is done, the underlying

units are those of work or energy, and that section should be looked at for

explanations concerning the calorie and Btu. In this section the (IT) values

have been used.

In this section it is the horsepower which provides confusion. Just like the

calorie, it can take 5 different values, and these are identified as necessary

by the addition of (boiler), (electric), (metric), (UK) and (water). Unlike the

calorie (whose 5 values are reasonably close to each other), the horsepower has

4 which are close and 1 (boiler) which is considerably different - it is about

13 times bigger than the others - but it seems to be very little used.

The S I unit of power is the watt. To change any of these other units of energy

or work into their equivalent values in watts use the operation and conversion

factor given. Those marked with # are exact. Other values are given to an

appropriate degree of accuracy.

Btu/hour x 0.293 071

Btu/minute              x 17.584 267

Btu/second              x 1055.056

calories/hour           x 0.001 639

calories/minute         x 0.069 78

calories/second         x 4.1868 #

ft lb-force/minute      x 0.022 597

ft lb-force/second      x 1.355 82

gigawatts [GW]          x 1 000 000 000

horsepower (electric)   x 746 #

horsepower (metric)     x 735.499

watts [W]               1

joules/hour             divide by 3600 #

joules/minute           divide by 60 #

joules/second           1

kilocalories/hour       x 1.163

kilocalories/minute     x 69.78

kg-force metres/hour    x 0.002 724

kg-force metres/minute  x 0.163 444

kilowatts [kW]          x 1000 #

megawatts [MW]          x 1 000 000 #

--------------------------------------------------------------------------------

Pressure or Stress

The S I unit of pressure is the pascal. The units of pressure are defined in

the same way as those for stress - force/unit area. To change any of these

other units of pressure (or stress) into their equivalent values in pascals use

the operation and conversion factor given. Those marked with # are exact. Other

values are given to an appropriate degree of accuracy. Measures based on water

assume a density of 1 kg/litre - a value which rarely matched in the real

world, though the error is small.

atmospheres x 101 325 #

bars                    x 100 000 #

centimetres of mercury  x 1333.22

centimetres of water    x 98.066 5 #

feet of water           x 2989.066 92 #

hectopascals [hPa]      x 100 #

inches of water         x 249.088 91 #

inches of mercury       x 3386.388

kg-force/sq.centimetre  x 98 066 5 #

kg-force/sq.metre       x 9.806 65 #

kilonewton/sq.metre     x 1000 #

kilopascal [kPa]        x 1000 #

kips/sq.inch            x 6 894 760

meganewtons/sq.metre    x 1 000 000 #

metres of water         x 9806.65 #

millibars               x 100 #

pascals [Pa]         1

millimetres of mercury  x 133.322

millimetres of water    x 9.806 65 #

newtons/sq.centimetre   x 10 000

newtons/sq.metre        1

newtons/sq.millimetre   x 1 000 000 #

pounds-force/sq.foot    x 47.880

pounds-force/sq.inch    x 6894.757

poundals/sq.foot        x 1.448 16

tons(UK)-force/sq.foot  x 107 251

tons(UK)-force/sq.inch  x 15 444 256

tons(US)-force/sq.foot  x 95 760

tons(US)-force/sq.inch  x 13 789 500

tonnes-force/sq.cm      x 98 066 500 #

tonnes-force/sq.metre   x 9806.65 #

--------------------------------------------------------------------------------

Speed

The S I compatible unit of speed is metres/second. To change any of these other

units of speed into their equivalent values in metres/second use the operation

and conversion factor given. Those marked with # are exact. Other values are

given to an appropriate degree of accuracy.

centimetres/minute divide by 6000 #

centimetres/second      divide by 100 #

feet/hour               divide by 11 811

feet/minute             x 0.005 08 #

feet/second             x 0.3048 #

inches/minute           divide by 2362.2

inches/second           x 0.0254 #

kilometres/hour         divide by 3.6 #

kilometres/second       x 1000 #

knots                   x 0.514 444

Mach number             x 331.5

metres/hour             divide by 3600 #

metres/minute           divide by 60 #

metres/second [m/s]  1

miles/hour              x 0.447 04 #

miles/minute            x 26.8224 #

miles/second            x 1609.344 #

yards/hour              divide by 3937

yards/minute            x 0.015 24 #

yards/second            x 0.9144 #

--------------------------------------------------------------------------------

Spread Rate (by mass)

The spread rate of a substance is a measure of how much of it there is covering

a unit area. The 'how much' can be measured by volume or by mass. The S I

compatible unit of spread rate by mass is kilograms/square metre. It is also a

measure of area density (mass/unit area) and is similar to - but not the same

as - pressure, which is force/unit area. For the rainfall conversions a density

of 1 kg/litre has been assumed. To change any of these other units of spread

rate into their equivalent values in kilograms/square metre use the operation

and conversion factor given. Those marked with # are exact. Other values are

given to an appropriate degree of accuracy. The conversion for rainfall assumes

a density of 1 kg/litre which is accurate enough for all practical purposes.

grams/sq.centimetre x 10 #

grams/sq.metre          divide by 1000 #

inches of rainfall      x 2.54

kilograms/hectare       divide by 10 000 #

kilograms/sq.centimetre x 10 000 #

milligrams/sq.metre     divide by 1000 #

millimetres of rainfall 1

kilograms/sq.metre   1

ounces/sq.foot          x 0.305 152

ounces/sq.inch          x 43.942

ounces/sq.yard          divide by 49.494

pounds/acre             divide by 8921.791

pounds/sq.foot          x 4.882 428

pounds/sq.inch          x 703.07

pounds/sq.yard          x 0.542 492

tonnes/hectare          divide by 10 #

tons(UK)/acre           divide by 3.982 942

tons(US)/acre           divide by 4.460 896

--------------------------------------------------------------------------------

Spread Rate (by volume)

The spread rate of a substance is a measure of how much of it there is covering

a unit area. The 'how much' can be measured by volume or by mass. The S I

compatible unit of spread rate by volume is cubic metres/square metre. However,

this is a rather large unit for most purposes and so litres/square metre is

often preferred. To change any of these other units of spread rate into their

equivalent values in litres/square metre use the operation and conversion

factor given. Those marked with # are exact. Other values are given to an

appropriate degree of accuracy.

cubic feet/acre divide by 142.913

cubic inches/sq.yard    divide by 51.024

cubic yards/sq.mile     divide by 3387.577

cubic metres/hectare    divide by 10 #

cubic metres/sq.km      divide by 1000 #

cubic metres/sq.metre   x 1000 #

fl. ounces(UK)/sq.yard  divide by 29.428

litres/square metre  1

gallons(UK)/acre        divide by 890.184

gallons(US)/acre        divide by 1069.066

gallons(UK)/hectare     divide by 2199.692

gallons(US)/hectare     divide by 2641.721

inches of rainfall      x 25.4 #

litres/hectare          divide by 10 000 #

millitres/sq.metre      divide by 1000 #

millimetres of rainfall 1

--------------------------------------------------------------------------------

Torque

The S I compatible unit of torque is the newton metre. To change any of these

other units of torque into their equivalent values in newton metres use the

operation and conversion factor given. Those marked with # are exact. Other

values are given to an appropriate degree of accuracy.

dyne centimetres        divide by 10 000 000 #

gram-force centimetres  x 0.000 098 066 5 #

kg-force centimetres    x 0.098 066 5 #

kg-force metres         x 9.806 65 #

newton centimetres      divide by 100 #

newton metres [Nm]   1

ounce-force inches      divide by 141.612

pound-force inches      x 0.112 984

pound-force feet        x 1.355 818

poundal feet            x 0.042 140

ton(UK)-force feet      x 3 037.032

ton(US)-force feet      x 2 711.636

tonne-force metres      x 9 806.65 #

--------------------------------------------------------------------------------